# AI-Based Fault Detection & Virtual Sensor Modeling – Feature Summary

Brief point-wise explanation of selected project features for presentation.

## 1) Sensor QoL / Sensor Health Monitoring

Continuously checks whether each sensor is working correctly.

Detects common sensor issues: drift (slow shift), bias (constant offset), stuck-at/flatline, noise spikes, missing data.

Outputs sensor status: Healthy / Suspect / Faulty.

Provides confidence score and recommended action (recalibrate / replace / ignore reading).

## 2) Explainable AI Output (XAI-lite)

Makes AI decisions transparent (important for safety-critical nuclear systems).

Instead of only “fault detected”, shows top sensors/features that influenced the decision.

Generates simple human-readable reason text (e.g., “Flow drop + pressure rise + abnormal temperature gradient”).

Implementation options: feature importance (tree models) or SHAP values for visualization.

## 3) Virtual Sensor Modeling (Soft Sensors)

AI-based virtual sensors estimate critical variables even when physical sensors fail.

Example virtual outputs: coolant flow rate, core outlet temperature, pressure differential, pump head.

Compares actual sensor vs virtual estimate: large deviation indicates possible sensor fault.

Outputs predicted value with a confidence range + mismatch warning.

## 4) Fault Classification + Severity Grading

Does not only detect faults; it diagnoses fault type and severity.

Fault type: sensor fault vs equipment fault vs process anomaly.

Severity levels: Minor / Medium / Critical.

Severity depends on deviation magnitude and persistence duration (longer + higher deviation → more severe).

## 5) Redundancy Voting & Confidence Fusion (2oo3 Concept)

Mimics real nuclear instrumentation logic using redundant sensors.

2oo3 voting: if 2 out of 3 sensors agree, that value is trusted.

Weighted fusion: sensors with poor health score are downweighted automatically.

Prevents a single faulty sensor from triggering false alarms or incorrect control actions.